COMPLIANCE PROCEDURES FOR DENTAL DEPARTMENT

The following general rules apply to the Dental Clinic and all divisions on the campus:

- Label every container, being particularly careful to label material that is poured over into a smaller bottle.  This smaller bottle must have a label written out in English words.

- Do not store any liquid material over the sink so that if the material were to leak it would run into the drain.

- Maintain all liquid materials in secondary containment and observe that the secondary containment trays are clean and dry.

- Segregate your cleaners even if they are just “household chemicals”.  These materials have chemical that can react just like the more concentrated chemicals.  A chart has been provided to keep similar chemicals together to prevent any accidental reaction.

- The departmental representative must perform inspections on the areas storing hazardous materials on a regular basis, paying attention to all aspects of the area including the storage of household chemicals.

The Dental Division also has materials, wastes and requirements that are specific just to this department.  The following general rules apply specifically to the to the Dental work areas in relation to hazardous materials and hazardous waste.

- The lead foil container must be labeled as a hazardous waste and the lid must be placed on the container and secured at all time unless in use.

- Amalgam must not be stored over the sink to prevent accidental discharge into the sink.

- The Pure Vac is a concentrated cleaner used to clean the dental vacuum system.  It is a “very strong acid” that is particularity hazardous to the skin, eyes and respiratory system.  Special effort must be made in storing and dispensing the liquid. The dispenser must be protected to assure that it does not leak onto the shelve, onto persons or about the clinic. This chemical must always be returned to a secondary containment tray,  assuring that the dispenser nozzle points into the tray so that it does not leak elsewhere.

- The fixer and developer are managed as hazardous waste and have very strict management rules.

  - Assure that the waste containers have the accumulate start written on them.  The accumulation start date is the date when the first drop of a material is placed into an empty container.  It begins a regulatory timeline where the waste must be disposed.

  - These virgin chemicals and the wastes must stored in secondary containment and must never overflow.  The secondary containment is regularly inspected to confirm clean dry containers.

- Identify the linen that has been used and may have body fluids on them as soiled linen, as separate from the clean linen that has not been used. Provide an identified area for the collection of soiled linen prior to washing.